NEW:

https://www.printables.com/model/499102-ikea-snudda-motorized-turntable-with-arduino

OLD:

Connect the motor to the controller

- The motor has 4 wires: A+, A- (one coil) B+, B- (second coil)

  - Connect wires to stepper motor terminal block on controller board

- Use a barrel jack breakout to connect the power supply to VCC and GND input on the controller board

- Plug the USB cable from the board to my computer

  - Board should appear as /dev/ttyUSB0

  - Install driver if board not appearing

Connect the motor to the plate

- Mount the hub onto the stepper motor shaft:

  - Align the flat side of the D-shaft with the set screw

  - Tighten it firmly using a hex key

- Attach the plate to the hub:

  - Align your plate’s drilled holes with the hub’s

  - Screw in using M3 bolts (ideally with thread lock for vibration resistance)

- Secure the motor:

  - If needed, mount the stepper motor itself to a fixed base using its M3 mounting holes