## Setup

Equipment

- Motorized lazy susan

  - https://www.printables.com/model/499102-ikea-snudda-motorized-turntable-with-arduino

- Stands (light, speaker, tripod etc)

- Rubber/Polystyrene head (10)

- Binaural mics

  - Dpa 4066: 800, omnidirectional

  - Dpa 4560: 900, binaural

- Speaker

  - Adam t5v (159)

- Zoom h4 audio interface

- Arduino - usb cable

- USB extension cord

- Mic power source

- Speaker power source

- Power extension cord

- PVC pipes and elbows

  - 5 each, ~70 total?

    - b&q

- Gorilla tape (10)

- Thick blanket (20)

  - Alternate: compressed neoprene

- Absorbent panels

  - Myworld has 25

Recording station setup

- Place the lazy susan on the ground

- Connect arduino to computer using usb cord + extension

- Place stand, extend to average human height (165 cm)

- Place head on stand

- Place mics around head, ear level

  - Connect to power

- Place speaker near mouth

  - Connect to power

Maze Setup

- How to elevate the entire maze?

  - Array of stands

    - How to move lazy susan through stands…

    - If clamps are strong enough, only place stands on 4 ends and center

- Build entire base out of sound absorbing panels (auralex max wall)

  - Can’t do this because of rotation device that mic/speaker lie on

- How to designate the end?

  - Special celebratory sound when landing in that space

- Padded canopy?

  - PVC pipes and elbows

## Measurement Recording

- Record the center, then near the intersections (5 points for each unit)

- 5x5 grid

- Integrate rotation and playing

  - Python or arduino

- Visualization of wav file

## Post-processing

## Future extension

- Use ambisonic microphone to build sphere of sound

  - ex Zoom H3-VR

- https://leomccormack.github.io/sparta-site/docs/plugins/sparta-suite/#matrixconv